National Textile University, Faisalabad

Department of Computer Science

| Name: | Malaika Arshad |
| --- | --- |
| Class: | BSCS-A 5th Semester |
| Registration No: | 23-NTU-CS-1043 |
| Assignment: | 1 |
| Course Name: | Embedded IoT Systems |
| Submitted To: | Sir Nasir |
| Submission Date: | 19-10-2025 |

Assignment 1

Question 1 — Short Questions

- Volatile for ISR variables:
volatile is used so the compiler doesn’t optimize the variable. Since an ISR can change it at any time, the CPU must always read its latest value from memory.

- Hardware-timer vs delay() debouncing:
A timer-ISR debounce is accurate and non-blocking, while delay() blocks the code and can slow down the whole program.

- IRAM_ATTR meaning:
IRAM_ATTR forces the ISR code into internal RAM so it can run fast and safely during interrupts, even if flash memory is temporarily unavailable.

- LEDC terms:
A channel connects a pin to PWM, a timer sets base PWM frequency/resolution, and duty cycle is how long the signal stays ON in one cycle.

- Avoid Serial in ISRs:
Serial prints are slow and can lock interrupts. ISRs should run quickly and exit fast.

- Advantages of timer-based scheduling:
Tasks run on precise time intervals without blocking loop(), giving smoother and more predictable behavior.

- I²C SDA and SCL:
SDA carries data, and SCL provides the clock signal that syncs communication.

- Polling vs interrupt input:
Polling keeps checking in a loop (less efficient), while interrupts react instantly only when an event happens.

- Contact bounce:
A button flips rapidly between HIGH and LOW when pressed. Debouncing is needed to avoid fake multiple presses.

- How LEDC improves PWM:
The LEDC hardware uses high resolution timers, making the PWM smoother and more precise than software PWM.

- ESP32 hardware timers:
The ESP32 has 4 hardware timers (per core: 2 each).

- Timer prescaler:
A prescaler slows down the input clock so we can get the exact timing we want.

- Duty cycle vs frequency:
Duty cycle is the ON percentage of one period. Frequency is how many PWM cycles run in one second.

- Calculating duty for brightness:
Duty = (brightness / max_value) × max_duty. For example, 50% brightness gives 50% duty.

- Non-blocking vs blocking timing:
Blocking (like delay()) stops the CPU. Non-blocking (like millis()) lets other code run at the same time.

- LEDC resolution:
ESP32 LEDC can go up to 1–20 bits, commonly 8–16 bits for PWM.

- Hardware timers vs LEDC timers:
General timers handle interrupts/precision timing, while LEDC timers are made specifically for PWM generation.

- SSD1306 vs GFX libraries:
Adafruit_SSD1306 controls the OLED display hardware. Adafruit_GFX provides the graphics functions like lines, fonts, shapes.

- Speeding up OLED text rendering:
Draw only changing parts, avoid clearing the whole screen, and use a smaller font.

- NodeMCU-32S specs (short):
Dual-core ESP32, Wi-Fi + Bluetooth, 4MB Flash, 34 GPIOs, hardware PWM, timers, and built-in ADC/DAC.

Question 2 — Logical Questions

- 10 kHz signal, ON = 10 ms — duty cycle:
Frequency 10 kHz ⇒ period = 1 / 10000 = 0.0001 sec = 0.1 ms
Duty = (ON time / Period) × 100 = 10 ms / 0.1 ms = 10,000%
(This means the given ON time doesn’t match a 10 kHz signal — it’s impossible because 10 ms is way longer than the period.)

- How many interrupts and timers can run at once:
ESP32 can use many interrupt sources, but it has 4 hardware timers, so up to 4 timer-based interrupts at the same time.

- PWM devices with different frequencies:
ESP32 has 4 LEDC timers, so you can have 4 different PWM frequencies. All channels sharing one timer must use the same frequency.

- 30% duty at 8-bit vs 10-bit:
Both are 30%, but 10-bit has finer steps (1024 levels) compared to 8-bit (256 levels). So brightness is smoother at 10-bit.

- Characters on 128×64 OLED:

  - At smallest 6×8 font → 128/6 × 64/8 ≈ 21 × 8 = about 168 characters

  - At largest 8×16 font → 128/8 × 64/16 = 16 × 4 = 64 characters