National Textile University, Faisalabad

Department of Computer Science

| Name: | Malaika Arshad |
| --- | --- |
| Class: | BSCS_A 5th Semester |
| Registration No: | 23-NTU-CS-1043 |
| Assignment : | Project |
| Course Name: | Embedded IOT |
| Submitted To: | Sir Nasir |
| Submission Date: | 26-10-2025 |

Project Documentation: Multi-Task ESP32 Control System

1. Project Overview

This project implements two distinct tasks—LED mode cycling (Task A) and press-type detection (Task B)—on a single ESP32 Dev Module. The system is controlled by two pushbuttons and provides visual feedback via an OLED display and audible feedback via a buzzer.

| Component | Quantity | Purpose |
| --- | --- | --- |
| ESP32 Dev Module | 1 | Microcontroller running the logic. |
| OLED Display (SSD1306, I²C) | 1 | Displays the current mode and button event. |
| Push Button | 2 | Inputs: one for mode cycle/reset, one for press detection. |
| LED (Red, Green, Blue) | 3 | Visual outputs for mode sequences and toggling. |
| Buzzer | 1 | Audible output for the long-press event. |

TASK A:

Circuit Diagram: Design a Wokwi circuit and draw a neat hand-sketch including:

• 2 push buttons

• 3 LEDs

• 1 buzzer

• 1 OLED

Task A — Coding: Use one button to cycle through LED modes (display the current state on

the OLED):

1. Both OFF

2. Alternate blink

3. Both ON

4. PWM fade

Use the second button to reset to OFF.

TASK A  :FIXED BUTTON ISSUE

WOKWI LINK  :

https://wokwi.com/projects/445501799320005633

SCREENSHOT:

Both OFF:

Alternate blink:

Code Of The Program:

#include <Wire.h>

#include <Adafruit_GFX.h>

#include <Adafruit_SSD1306.h>

#define SCREEN_WIDTH 128

#define SCREEN_HEIGHT 64

Adafruit_SSD1306 display(SCREEN_WIDTH, SCREEN_HEIGHT, &Wire, -1);

// Pins

#define LED1 14

#define LED2 27

#define LED3 26

#define MODE_BUTTON 25

#define RESET_BUTTON 33

#define BUZZER 19

int modeState = 0;

unsigned long previousMillis = 0;

int brightness = 0;

int fadeAmount = 5;

void showMode() {

display.clearDisplay();

display.setTextSize(2);

display.setTextColor(SSD1306_WHITE);

display.setCursor(0, 0);

if (modeState == 0) display.println("Both OFF");

if (modeState == 1) display.println("Alt Blink");

if (modeState == 2) display.println("Both ON");

if (modeState == 3) display.println("PWM Fade");

display.display();

}

void setup() {

pinMode(LED1, OUTPUT);

pinMode(LED2, OUTPUT);

pinMode(LED3, OUTPUT);

pinMode(MODE_BUTTON, INPUT_PULLUP);

pinMode(RESET_BUTTON, INPUT_PULLUP);

pinMode(BUZZER, OUTPUT);

display.begin(SSD1306_SWITCHCAPVCC, 0x3C);

showMode();

}

void loop() {

// MODE change button

if (digitalRead(MODE_BUTTON) == LOW) {

modeState = (modeState + 1) % 4;

tone(BUZZER, 700, 80);

showMode();

delay(250);

}

// RESET button

if (digitalRead(RESET_BUTTON) == LOW) {

modeState = 0;

tone(BUZZER, 400, 80);

showMode();

delay(250);

}

// MODE Behaviors

if (modeState == 0) {

digitalWrite(LED1, LOW);

digitalWrite(LED2, LOW);

digitalWrite(LED3, LOW);

}

if (modeState == 1) {

if (millis() - previousMillis >= 500) {

previousMillis = millis();

digitalWrite(LED1, !digitalRead(LED1));

digitalWrite(LED2, !digitalRead(LED1));

digitalWrite(LED3, LOW);

}

}

if (modeState == 2) {

digitalWrite(LED1, HIGH);

digitalWrite(LED2, HIGH);

digitalWrite(LED3, HIGH);

}

if (modeState == 3) {

analogWrite(LED1, brightness);

analogWrite(LED2, brightness);

analogWrite(LED3, brightness);

brightness += fadeAmount;

if (brightness <= 0 || brightness >= 255) fadeAmount = -fadeAmount;

delay(15);

}

}

TASK B:

— Coding: Use a single button with press-type detection (display the event on the OLED): • Short press → toggle LED • Long press (> 1.5 s) → play a buzzer tone.

WOKWI LINK:

https://wokwi.com/projects/445896892712775681

WORKING SCREENSHOT:

LED ON:

Buzzer running:

Code of the program:

#include <Wire.h>

#include <Adafruit_GFX.h>

#include <Adafruit_SSD1306.h>

#define SCREEN_WIDTH 128

#define SCREEN_HEIGHT 64

Adafruit_SSD1306 display(SCREEN_WIDTH, SCREEN_HEIGHT, &Wire, -1);

// Pins

#define BUTTON 25

#define LED 14

#define BUZZER 19

// Variables

bool ledState = false;

unsigned long pressTime = 0;

bool buttonPressed = false;

const unsigned long longPressTime = 1500; // 1.5 seconds

void showOLED(const char* msg) {

display.clearDisplay();

display.setTextSize(2);

display.setTextColor(SSD1306_WHITE);

display.setCursor(0, 0);

display.println(msg);

display.display();

}

void setup() {

pinMode(BUTTON, INPUT_PULLUP);

pinMode(LED, OUTPUT);

pinMode(BUZZER, OUTPUT);

display.begin(SSD1306_SWITCHCAPVCC, 0x3C);

showOLED("Ready");

}

void loop() {

int buttonState = digitalRead(BUTTON);

if (buttonState == LOW && !buttonPressed) {

buttonPressed = true;

pressTime = millis(); // record press time

}

if (buttonState == HIGH && buttonPressed) {

buttonPressed = false;

unsigned long pressDuration = millis() - pressTime;

if (pressDuration < longPressTime) {

// Short press

ledState = !ledState;

digitalWrite(LED, ledState);

showOLED("Short");

}

else {

// Long press

showOLED("Long");

tone(BUZZER, 800, 600); // play tone for 0.6s

}

}

}